【IMPORTANT】：

While updating firmware，please do not switch off power。

And please plug in to USB 2.0 port (suggest).

ASUS Xonar Essence One USB Firware Update：

- After Xonar Essence One is powered on and connected to PC，execute FWUpdateEndUser.exe

- Select “Device Name”as “USB\Vid_0b05&Pid_17a8&Rev_xxxx”

- Select “Update FW” and choose the latest Firmware document(*.hex). Press ”OK”and it will automatically update USB Firmware

- Here’s the loading process

- After firmware has been updated successfully, you will see the following message “Firmware Update Finished. Please replug the device.”

- Restart Xonar Essence One and check if it is the latest version

- FAQ

【QUESTION】

Under Windows 7 , if firmware updates unsuccessfully, it cannot update correctly. Can’t find the device under Device manager.

【ANS】

Follow the steps in this order:

- Enter control panel > Device manager, find Universal Serial Bus controllers’ “USB Composite Device”

- Right click to select “Update Driver Software”

- Select ”Browse my computer for driver software”

- Locate to ”\Xonar Essence One USB FW Update \ Driver”and select “Install ”

- After the installation is complete, the following screen will appear。

- Execute“FWUpdateEndUser.exe” and select“Device Name”as“USB\Vid_0D8C&Pid_5200&Rev_0100”

- Reupdate firmware as step 2 .